PROJECT-1

AI BASED FAULT DETECTION IN DIGITAL CIRCUIT

The core idea — digital circuits can have manufacturing defects or runtime failures. The most common model is "stuck-at faults" (a signal stuck at 0 or 1 regardless of input). Your AI model learns to detect these by analyzing the difference between expected and actual circuit outputs.

Basic code of full adder in python

import pandas as pd

data = []

truth_table =[

[0,0,0,0,0],

[0,0,1,1,0],

[0,1,0,1,0],

[0,1,1,0,1],

[1,0,0,1,0],

[1,0,1,0,1],

[1,1,0,0,1],

[1,1,1,1,1]

]

for row in truth_table:

A,B,Cin,Sum,Cout = row

# No fault

data.append([A,B,Cin,Sum,Cout,"No Fault"])

# Sum stuck at 0

data.append([A,B,Cin,0,Cout,"Sum Stuck-at-0"])

# Sum stuck at 1

data.append([A,B,Cin,1,Cout,"Sum Stuck-at-1"])

# Cout stuck at 0

data.append([A,B,Cin,Sum,0,"Cout Stuck-at-0"])

# Cout stuck at 1

data.append([A,B,Cin,Sum,1,"Cout Stuck-at-1"])

df = pd.DataFrame(data, columns=[

"A","B","Cin","Sum","Cout","Fault"

])

df.to_csv("dataset.csv", index=False)

print(df.head())

print("Dataset created successfully!")

CHECKING ACCURACY (1.0)

import pandas as pd

from sklearn.tree import DecisionTreeClassifier

from sklearn.metrics import accuracy_score

import joblib

# Load dataset

df = pd.read_csv("dataset.csv")

# Features and labels

X = df[["A","B","Cin","Sum","Cout"]]

y = df["Fault"]

# Train model on full dataset

model = DecisionTreeClassifier(random_state=42)

model.fit(X, y)

# Predict on same data

y_pred = model.predict(X)

# Accuracy

accuracy = accuracy_score(y, y_pred)

print("Accuracy:", accuracy)

USER INPUT AND THEN COMPUTER TELLING FAULT

import joblib

import pandas as pd

# Load trained model

model = joblib.load("fault_model.pkl")

# User input

A = int(input("Enter A: "))

B = int(input("Enter B: "))

Cin = int(input("Enter Cin: "))

Sum = int(input("Enter observed Sum: "))

Cout = int(input("Enter observed Cout: "))

# Create dataframe with feature names

input_data = pd.DataFrame(

[[A,B,Cin,Sum,Cout]],

columns=["A","B","Cin","Sum","Cout"]

)

# Prediction

prediction = model.predict(input_data)

print("Detected Fault:", prediction[0])

We are building a system that:

- Takes circuit inputs

- Observes outputs

- Checks whether outputs are correct

- Detects faults automatically using ML

It is like an intelligent testing system for digital circuits.

# STEP 1 — Understanding Full Adder

A full adder adds 3 binary bits:

- A

- B

- Cin (Carry input)

Outputs:

- Sum

- Cout (Carry output)

# Example

Suppose:

A = 1

B = 0

Cin = 1

Binary addition:

1 + 0 + 1 = 10

In binary:

- right digit → Sum

- left digit → Carry

So:

Sum = 0

Cout = 1

# FULL ADDER EQUATIONS

## Sum Equation

Sum=A⊕B⊕CinSum = A \oplus B \oplus CinSum=A⊕B⊕Cin

XOR means:

- same bits → 0

- different bits → 1

## Carry Equation

Cout=AB+BCin+ACinCout = AB + BCin + ACinCout=AB+BCin+ACin

Meaning:
 Carry becomes 1 if at least two inputs are 1.

# STEP 2 — Truth Table

This is the correct behavior of a full adder.

| A | B | Cin | Sum | Cout |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

This table is the “ideal circuit.”

# STEP 3 — Understanding Faults

Real circuits can fail.

Example:

- wire broken

- transistor damaged

- short circuit

- output permanently fixed

One common fault:

# Stuck-at Fault

Meaning:
 Output gets stuck permanently at:

- 0
 OR

- 1

# Example of Fault

Correct output:

Sum = 0

But due to fault:

Sum = 1

This is:

Sum Stuck-at-1

# STEP 4 — Dataset Creation

This part:

data.append([A,B,Cin,Sum,Cout,"No Fault"])

means:

Store:

- inputs

- outputs

- label

Example row:

| A | B | Cin | Sum | Cout | Fault |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 1 | No Fault |

# Faulty Rows

Suppose the correct Sum is 0.

This line:

data.append([A,B,Cin,1,Cout,"Sum Stuck-at-1"])

creates artificial faulty data.

Example:

| A | B | Cin | Sum | Cout | Fault |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 1 | 1 | Sum Stuck-at-1 |

Meaning:
 Circuit incorrectly produced Sum=1.

# WHY WE CREATE MANY ROWS

Machine learning learns patterns.

We show examples like:

- “When this output changes this way → this fault occurs.”

This is training.

# STEP 5 — Understanding Features and Labels

This code:

X = df[["A","B","Cin","Sum","Cout"]]

means:
 These are INPUT FEATURES for AI.

AI observes:

- A

- B

- Cin

- observed Sum

- observed Cout

This line:

y = df["Fault"]

means:
 This is the ANSWER AI should learn.

Example:

Inputs → Fault Type

# STEP 6 — Training the Model

model.fit(X, y)

This is where learning happens.

The Decision Tree studies patterns like:

IF:

Expected Sum should be 0

but observed Sum is 1

THEN:

Likely Sum Stuck-at-1

# STEP 7 — Prediction Phase

User enters:

A = 1

B = 0

Cin = 1

Observed Sum = 1

Observed Cout = 1

Program asks:

## “Is this output correct?”

Correct output should be:

Sum = 0

Cout = 1

But observed Sum became 1.

So model predicts:

Sum Stuck-at-1

# WHAT ARE “OBSERVED OUTPUTS”?

VERY IMPORTANT.

These values:

Observed Sum

Observed Cout

represent:

## actual outputs coming from the real circuit.

Maybe:

- from hardware

- from simulation

- from sensors

- from logic analyzer

Our system compares:

## Expected vs Observed

and detects faults.

# WHY THIS IS CALLED AI-BASED

Because:

- instead of manually writing many if-else conditions,

- the model learns patterns automatically from data.

# Decision Tree Simple Idea

Decision tree works like:

Is Sum wrong?

YES →

Is Sum = 1?

YES → Sum Stuck-at-1

It creates intelligent decision rules automatically.